Current and Charge

Learning Objectives

- Recognise circuit symbols and use them to build and draw circuits

- Use and rearrange the current, charge, time equation

Current and Charge

Electric current is the flow of charge. In wires, this charge is carried by electrons moving around the circuit.

Electrons carry a negative charge. They move freely between the positive ions in a metal.

The size of an electric current is the rate of flow charge:

Charge flow = current x time

Q = I x t

Current flows from the + of the power supply to the – of the power supply in a circuit.

Ammeters measure the current in amps (A) and are connected in series with the components in a circuit.

Resistors

Variable resistors allows us to control the current in a circuit.

Diodes

Diodes only allow current to flow in one direction (the direction of the arrow on the symbol).

Info

Readings of current are often less than 1 amp, so currents are often given in milliamps, (mA).

1mA = 1x10^-3 A=0.001A

Voltmeters measure potential difference across a component and are connected in parallel with the component.

Potential Difference

Learning Objectives

- Explain and calculate potential difference

- Explain and calculate resistance

- State and Explain Ohm’s Law

- Describe the effect of reversing the current through a component

PD

PD is a measure of the amount of work done (energy transferred) by each coulomb of charge passing through a component.

PD across a component = work done / charge

V = W / Q

PD is measured in volts (V) or joules per coulomb (J/C).

1V = 1J/C

Resistance

The positively charged ions in a wire vibrate. As electrons move past and between these ions, they collide with them and with each other which makes it difficult for them to pass.

This difficulty in passing is what causes resistance. The higher the resistance, the harder it is to make a current flow through.

Resistance = PD / current

R = V / I

Resistance is measured in ohms or volts per ampere (V/A)

Resistors, and other components with resistance, limit the current in a circuit (for any given PD).

Ohm’s Law

Components which obey Ohm’s law produces a straight line graph, through the origin, when current is plotted against PD.

Current PD Graph for a Filament Lamp

The graph is not a straight line through the origin. The current is not proposional to the PD. The lamp does not obey Ohm’s law.

The lamp gets hotter as the current increases. As the lamps get hotter, the resistance goes up (the gradient gets less steep, gradient = R). The effect is the same whichever way the current is flowing.

Current PD Graph for a Diode

The current PD graph for a diode curves, so the current is not proportional to PD. It does not obey ohm’s law. The current PD graph for a diode shows that the resistance in the reverse bias directiom (current trying to flow the wrong way) is very high and almost no current is let through. Devices which obey Ohm’s Law are called ohmic conductors. Devices which do not obey Ohm’s Law are non-ohmic conductors.

Variable Resistors

Variable resistors can be adjusted to change their resistance. They can be used to vary the current in a circuit. By putting a variable resistor in series with a lamp, the lamp can be made brighter (when there is less resistance) or dimmer (when there is more resistance).

Continued on Resistance, Current and PD

Learning Objectives

- Describe the behaviour of a thermistor and an LDR

- Describe the effect of resistance on current

- State what happens to current + PD in a series circuit

Current PD graph for a thermistor

As the temperature goes down, the resistance goes up and visa-versa.

A thermistor is a resistor whose resistance varies with temperature. As temperature goes up, its resistance goes down. As temperature goes down, its resistance goes up.

Current PD graph for an LDR

A light dependant resistor is an LDR.

An LDR is a resistor whose resistance varies with light intensity. As light intensity (brightness) goes up, its resistance goes down. As light intensity goes down, is resistance goes up.

Series Circuits

The current is the same at all points in a series circuit.

The total PD of the power supply is shared between the components in a series circuit. If we were to add cells in series, connected to act in the same direction, their PDs add up.

If we mix their directions, their PDs add in one direction and subtract in the other.

The Resistance of Components in Series

The PD is shared between resistors (or other components in series) in the same ratio as their resistances.

For 3 or more resistors, the rule is the same.

V1:V2:V3 = R1:R2:R3

Calculate the current through the first resistor and the current through the second resistor.

If we replaced the two resistors with a single resistor which gave a current of 1.0A, when the PD of the power supply was 15V, its resistance would be 15 ohms (15/1 = 15).

The two resistors have the values 10 ohms and 5 ohms.

10 + 5 = 15

So we see from this that resistance in series add up.

For resistors or resistances in series, the general rule is:

Rtotal = R1 + R2 + R3 + …

How does resistance of a wire vary with length?

Both PD and Resistance, increase, while Current decreases.